【内核配置文件】

通过将arch/arm/configs/里的suniv开头的配置文件改名，执行./build.sh时候，会报错。

来知道，内核编译使用的默认配置文件是sunivw1p1smp_defconfig

那么我们先将该文件备份一下。

进入linux-3.10目录，

make ARCH=arm sunivw1p1smp_defconfig

然后，make ARCH=arm menuconfig进行配置

配置完毕并确认功能正常，将内核根目录下的.config文件，替换

arch/arm/configs/sunivw1p1smp_defconfig

【USB支持】

- U盘和读卡器的识别和挂载

make ARCH=arm menuconfig进入内核配置，

- Device Drivers  --->

- [*] USB support  --->

- <*>     USB Mass Storage support

- 需要将U盘或读卡器在window上，使用PhoenixCard执行(恢复卡)操作变为一个分区后，插入板子，可以在/dev/目录下生成sdx文件，这就是U盘或TF卡。可以使用mount命令进行挂载。

- 比如我在/mnt目录创建一个文件夹用于挂载

- mkdir /mnt/usb1

- 然后将生成的/dev/sda挂载到usb1上。

- mount /dev/sda /mnt/usb1

- 卸载只需要

- umount /mnt/usb1

- 如果在/dev目录生成了多个sdx sdx1 sdx2，那是该U盘或读卡器内有很多分区。

- 在编写U盘挂载和读写程序时，要注意插入的U盘需要考虑分区问题。

- 如果挂载时报ISO 8859-1的错误，那么就配置一下本地语言支持

File systems  --->

-*- Native language support  --->

<*>   NLS ISO 8859-1  (Latin 1; Western European Languages)

- USB转串口

- make ARCH=arm menuconfig进入内核配置，

- Device Drivers  --->

- <*>   USB Serial Converter support  --->

- <*>   USB Winchiphead CH341 Single Port Serial Driver

- <*>   USB CP210x family of UART Bridge Controllers

- <*>   USB Prolific 2303 Single Port Serial Driver

- 打开内核配置后

- 插上CP2102，能在/dev下识别为ttyUSB0

- 插上CH340，能在/dev下识别为ttyUSB0

- USB摄像头

- make ARCH=arm menuconfig进入内核配置，

- Device Drivers  --->

- <*> Multimedia support  --->

- [*]   Cameras/video grabbers support

- [*]   Media USB Adapters  --->

- <*>   USB Video Class (UVC)

- [*]     UVC input events device support (NEW)

- 因为我这测试的USB摄像头是PC的摄像头，上面的驱动芯片是SN9C1开头的，所以我把这个也配置上。

- <*>   USB SN9C1xx PC Camera Controller support (DEPRECATED)

- 插上摄像头，能在/dev下识别为video0

- USB转网口

- make ARCH=arm menuconfig进入内核配置，

- Device Drivers  --->

- [*] Network device support  --->

- USB Network Adapters  --->

- <*> USB Pegasus/Pegasus-II based ethernet device support

- <*> Realtek RTL8152 Based USB 2.0 Ethernet Adapters

- <*> Multi-purpose USB Networking Framework

- <*>   ASIX AX88xxx Based USB 2.0 Ethernet Adapters

- 插上绿联带网口HUB(AX88772B方案)，可以识别网卡，但udhcpc无法获得ip。

- 使用小米USB网卡，能识别并能udhcpc获取IP，

- 经过测试发现，F1C200S对USB HUB的支持不好，所以尽量不要使用HUB。